How to reset the Linak lift up and down stroke. (rev 2)

- Press the down arrow on the wired pad and wait until the stroke is all the way down.

- Press the down arrow again on the wired pad for 5 second to reset.

- Press the up arrow on the wired pad and wait until the stroke is all the way up.

- Press the down arrow on the wired pad and wait until the stroke is all the way down.

- Press the up button on the remote and wait until the lift is all the way up.

- Press the down button on the remote and wait until the stroke is all the way down.

- Reset is done.